.

## Poultry House  Monitor  Log: DHT22 + LDR Sensor Setup

Date: July 9, 2026

### What was accomplished today

1. Wokwi simulation

- Set up Arduino Uno + breadboard + DHT22 + LDR in Wokwi

- Wired DHT22 (VCC/GND → rails, DATA → D2) and confirmed real-time temperature/humidity readings via Serial Monitor

- Identified and corrected an LDR wiring error — initially wired the LDR and resistor in parallel (non-functional as a divider); corrected to a proper series voltage divider with a shared junction column feeding into A0

2. LDR voltage divider — understanding and correct wiring

- Confirmed the LDR in hand is a bare 2-leg photoresistor, requiring a manually built voltage divider (not a pre-assembled module)

- Correct wiring: LDR leg 1 → +5V rail, LDR leg 2 + resistor leg 1 share one column (the junction), resistor leg 2 → GND rail, junction → A0

- Worked through the underlying electrical principle: the junction voltage is the equilibrium point between the LDR's light-dependent resistance and the fixed resistor's constant resistance

3. Physical breadboard build

- Wired DHT22 (VCC, GND, DATA → D2) on real hardware

- Wired the LDR + resistor voltage divider (→ A0) on real hardware, matching the corrected simulated layout

4. Working sketch (DHT22 + LDR only)

#include <DHT.h>

#define DHTPIN 2
#define DHTTYPE DHT22
DHT dht(DHTPIN, DHTTYPE);

int ldrPin = A0;

void setup() {
  Serial.begin(9600);
  dht.begin();
}

void loop() {
  float temperature = dht.readTemperature();
  float humidity = dht.readHumidity();
  int lightLevel = analogRead(ldrPin);

  if (isnan(temperature) || isnan(humidity)) {
    Serial.println("Failed to read from DHT22 sensor!");
  } else {
    Serial.print("Temp: ");
    Serial.print(temperature);
    Serial.print(" C | Humidity: ");
    Serial.print(humidity);
    Serial.print(" % | Light: ");
    Serial.println(lightLevel);
  }

  delay(2000);
}

### Next steps

- Add relay + fan actuator (D8) with hysteresis-based temperature control

- Test MQ-135 ammonia/air-quality sensor integration

- Add LCD/LED/buzzer for local status display